xxxxxxxxx

SECTION 1:  RAWINSONDE

List of suspect stations

Experimental Procedure

Data Monitored:          All TEMPS and PILOTS, 00 & 12 UTC

Standard of Comparison:  Global Data Assimilation System

Six-hour Forecast.

Geopotential Heights:    Units:  meters

Wind Vectors:            Units:  meters per second

Selection Criteria:      Number of reports greater than or equal to 10.

Geopotential: Normalized Root-Mean-Square

greater than or equal to 100

meters, all levels 700-50hPa.

Wind Vector:  Root-Mean-Square greater than

or equal to 14.5 mps, all levels.

Gross Error Limit:

Level(hPa)    Geopotential    Wind Vector    Normalizing Constant

1000            100              35               ----

850            100              35               ----

700            100              40               2.90

500            150              45               2.20

400            175              50               1.90

300            200              60               1.60

250            225              60               1.50

200            250              50               1.37

150            275              50               1.19

100            300              45               1.00

50            325              40               0.80

Bias: Mean difference from assimilating forecast excluding gross errors

RMS:  Root-Mean-Square of differences from assimilating forecast excluding gross errors

# LEVELS >:  The number of levels exceeding the normalized root-mean-square value.

xxxxxxxxx

SECTION 2:  AIRCRAFT

Data Monitored:   Aircraft, AMDAR/ASDAR, and ACARS

temperature and wind reports, all report times.

(RECCO reports are not included)

Standard of Comparison:  Global Data Assimilation System

Six-hour Forecast.

Selection Criteria: Number of reports greater than or equal to 10.

Gross Error Limit:  Wind - Forecast:  40mps, all levels.

Temp - Forecast:  10 deg. C., all levels.

RMS:  Root-Mean-Square of differences from assimilating forecast.

Table column headers: OBS is total number of observations

(excluding GRS, the number of gross error reports) and

REJ is number of reports rejected by quality control.  Non-

gross REJ reports are included in the statistical calculation.

xxxxxxxxx

SECTION 3:  SATELLITE WINDS

Data Monitored:          All SATOBS, all report times.

Standard of Comparison:  Global Data Assimilation System

Six-hour Forecast.

Selection Criteria:      Number of reports greater than or equal to 10.

Gross Error Limit:       Wind - Forecast:  60mps, all levels.

Data Selected:           Layer:            1000-701hPa

700-401hPa

400-100hPa

Bias:                    Mean difference in wind speed (mps),

observation minus assimilating forecast.

RMS:                     Root-Mean-Square (mps) of differences from

assimilating forecast.

GRAPHS ONLY -- NO ELECTRONIC TRANSMISSIONS AVAILABLE

xxxxxxxxx

SECTION 4:  SATELLITE TEMPERATURES

Data Monitored:          All SATEMS, NOAA12, NOAA14

all report times, 240km resolution.

A:  Clear Path Retrievals

B:  Cloudy Retrievals

C:  Microwave Retrievals

Standard of Comparison:  Global Data Assimilation System

Six-hour Forecast.

Selection Criteria:      All SATEM sounding profiles over water.

Gross Error Limit:       None.

Data Selected:           Layer Thickness:   1000-850hPa

300-100hPa

100-50hPa*

Bias:                    Mean difference in layer mean virtual temperature,

observation minus assimilating forecast.

*  Temporary values, 50-30hPa planned.

*** DATA NOT AVAILABLE ***

RETRIEVAL DATA WILL BE REPLACED BY RADIANCE DATA

BUT WILL NOT BE AVAILABLE ELECTRONICALLY.

xxxxxxxxx

SECTION 5: SURFACE MARINE REPORTS

Data Monitored:          Ship, CMAN, and Buoy reports, all times.

Standard of Comparison:  Global Data Assimilation System or Aviation

Six-hour Forecast.

Ship, CMAN, and Buoy History.

Selection Criteria:      Pressure:    20 or more reports.

Absolute value of bias > 4 hPa.

Standard deviation > 6 hPa.

Wind Speed:  20 or more reports.

Abs. value of bias > 5 m/sec.

Wind Dir.:   20 or more reports.

Abs. value of bias > 30 deg.

Standard deviation > 80 deg.

Sea Surface

Temperature: 20 or more reports.

Absolute value of bias > 4.0 C.

Standard Deviation > 6.0 C.

Air

Temperature:  Same as SST

Number of

Gross Errors: > 25% of reports.

Gross Error Limit:       Pressure:  15hPa

Wind Speed: > 30 knots (15.45 m/sec).

Wind Direction: > 140 degrees.

Air Temperature: > 15.0 C.

Sea Surface Temperature: > 15.0 C.

xxxxxxxxx

SECTION 6:  ASAP SHIP REPORTS

Data Monitored:          All ASAP (Ship) reports, all times.

Standard of Comparison:  Global Data Assimilation System

Six-hour Forecast.

Gross Error Limit:

Level(hPa)    Geopotential    Wind Vector    Normalizing Constant

1000            100             35                ----

850            100             35                ----

700            100             40                2.90

500            150             45                2.20

400            175             50                1.90

300            200             60                1.60

250            225             60                1.50

200            250             50                1.37

150            275             50                1.19

100            300             45                1.00

50            325             40                0.80