NCAR/UNIDATA

CfRadial Data File Format

Proposed
CF-compliant netCDF Format

for Moments Data for RADAR and LIDAR

in Radial Coordinates

Version 1.0, draft 8

Mike Dixon, Wen-Chau Lee

Bob Rilling , Chris Burghart,
Joe Van Andel, Dennis Flanigan
EOL, NCAR

2010-11-01

1	Introduction	4

1.1	Purpose	4

1.2	Extensions to the CF convention	4

1.3	Strict use of variable and attribute names	4

1.4	_FillValue value attribute	5

1.5	Required vs. optional variables	5

2	Data Content Overview	5

2.1	The nature of radar and lidar moments data	5

2.2	Coordinate system for moments data arrays	5

2.3	Geo-reference variables	6

2.4	Sweep indexes – a "pseudo" third dimension	7

2.5	Constant range geometry per volume	7

2.6	No grid mapping variable	7

2.7	Calibration information	8

2.8	Compression	8

3	Convention hierarchy	9

4	CF/Radial base convention	10

4.1	Global attributes	10

4.2	Dimensions	11

4.3	Global variables	11

4.4	Coordinate variables	12

4.4.1	Attributes for time coordinate variable	12

4.4.2	Attributes for range coordinate variable	12

4.5	Location variables	13

4.6	Sweep variables	13

4.7	Sensor pointing variables	14

4.7.1	Attributes for azimuth(time) variable	14

4.7.2	Attributes for elevation(time) variable	15

4.8	Moving platform geo-reference variables	15

4.9	Moments field data variables	17

5	Sub-conventions	18

5.1	The instrument_parameters sub-convention	18

5.2	The radar_parameters sub-convention	19

5.3	The lidar_parameters sub-convention	20

5.4	The radar_calibration sub-convention	20

5.4.1	Dimensions	20

5.4.2	Variables	20

5.5	The lidar_calibration sub-convention	23

5.6	The platform_velocity sub-convention	23

5.7	The geometry_correction sub-convention	24

6	Standard names	26

6.1	Proposed standard names for metadata variables	26

6.2	Standard names for moments variables	33

7	Computing gate location from geo-reference variables	34

7.1	Symbol definitions	34

7.2	Stationary platforms	34

7.2.1	LIDARs	34

7.2.2	RADARs	35

7.3	Moving platforms	35

7.4	The geometry of moving platforms	36

7.4.1	Reference frame	36

7.4.2	Primary rotation axes	39

7.4.2.1	Type Z radars: primary axis za (normal radar, nose radar)	39

7.4.2.2	Type Y radars: primary axis ya (e.g. tail radar rotating around longitudinal axis of fuselage, ELDORA)	40

7.4.2.3	Type X radars, primary axis xa (e.g. belly radar scanning forward and aft, EDOP)	40

7.4.3	Rotating Xa to Xh	40

7.4.4	Computing earth-relative elevation and azimuth	42

8	Change log	43

8.1	Version 1.0, draft7: 2010-06-16	43

8.2	Version 1.0, draft8: 2010-11-01	43

9	References	45

# Introduction

## Purpose

The purpose of this document is to specify a CF-compliant netCDF format for radar and lidar moments data in radial (i.e. polar) coordinates.

The intention is that the format should, as far as possible, comply with the CF conventions for gridded data. However, the current CF 1.4 convention does not support radial radar/lidar data. Therefore, extensions to the conventions will be required.

The current CF conventions are documented at:

http://cf-pcmdi.llnl.gov/documents/cf-conventions/1.4

## Extensions to the CF convention

This convention introduces the following extensions to CF:

- The following axis attribute types:

- axis = "radial_azimuth_coordinate";

- axis = "radial_elevation_coordinate";

- axis = "radial_range_coordinate";

- Additional standard units. The following need to be added:

- dB (ratio of two values, in log units. For example, ZDR).

- dBm (power in milliwatts, in log units)

- dBZ (radar reflectivity in log units)

- Additional standard names.

CfRadial files will be CF compliant, with the above extensions.

## Strict use of variable and attribute names

However, because of the inherent complexity of radial radar and lidar data, the CfRadial format requires extra strictness, as compared to CF in general, in order to keep it manageable. There are so many metadata variables in CfRadial that it is essential to require strict adherence to the dimension names and metadata variable names exactly as specified in this document, in addition to their standard names. It is not practical to expect an application to search for standard names for metadata variables, since this makes the code unnecessarily complex.

Since this is a completely new format, there is no requirement to support legacy data sets which are less strictly defined.

Note that this strictness requirement only applies to metadata variables. The moments data fields will be handled as usual in CF, where the standard name is the definitive guide to the contents of the field. Suggested standard names for radar variables not yet supported by CF are listed in section 6.

One exception to this is the dimensions used to specify string variable lengths. String length dimensions may be added as needed. This document refers to the string length dimension as ‘string_length’, but any suitable dimension may be used in its place. See section 4.2.

## _FillValue value attribute

The CF documentation mentions that the missing_value attribute is deprecated, and that _FillValue should be used instead. However, the CF document also states that missing_value is supported for backward compatibility purposes.

CfRadial will use the _FillValue attribute to indicate missing values.

Note: the CF documentation mentions that the missing_value attribute is deprecated, and that _FillValue should be used instead. However, the CF document also states that missing_value is supported for backward compatibility purposes.

Therefore, it is recommended that software readers check for missing_value as well.

## Required vs. optional variables

If not otherwise stated, the inclusion of a variable is mandatory.

Some variables are optional. If so, this is stated in this document. If a variable is omitted, a reader should set the variable to missing everywhere.

# Data Content Overview

## The nature of radar and lidar moments data

As a radar or lidar scans (or points) the data fields (or moments) are produced over an entity specified by a time interval or angular interval.

We refer to this entity as a ray, beam or dwell. In this document we will use the term ray.

For a given ray, the field data are computed for a sequence of ranges increasing radially away from the instrument. These are referred to as range gates.

In most cases, the spacing between the range gates is constant along the ray, although this is not necessarily the case. For example, some NOAA radars have gate spacings of 75m, 150m and 300m. Therefore, we need to handle the cases for which the range gate spacing is variable.

## Coordinate system for moments data arrays

The moments data to be handled by this format is represented in 2 principal dimensions:

- time: rays have monotonically increasing time

- range: bins have monotonically increasing range

Figure 2.1 Data organization in time and range

The primary coordinate is time and the secondary coordinate is range.

The field data are stored in contiguous 1-D arrays, in which the range index varies the fastest, and the time index the slowest.

## Geo-reference variables

A subset of the metadata variables in CfRadial are used to locate a radar or lidar measurement in space.

These are:

- range

- elevation

- azimuth

- latitude

- longitude

- altitude

See sections 4.4, 4.5 and 4.7 for details on these variables.

For moving platforms, extra variables are required for geo-referencing.

These are:

- heading

- roll

- pitch

- rotation

- tilt

See section 4.8 for details on these variables.

The mathematical procedures for computing data location relative to earth coordinates are described in detail in section 7.

## Sweep indexes – a "pseudo" third dimension

A set of two or more related sweeps, typically a complete 3-D radar or lidar scan, is referred to as a volume.

A volume scan is comprised of one or more sweeps.

Scanning may be carried out in a number of different ways. For example:

- horizontal scanning at fixed elevation (PPI mode)

- vertical scanning at constant azimuth (RHI mode)

- antenna not moving, i.e. constant elevation and azimuth (staring or pointing)

- aircraft radars which rotate around the longitudinal axis of the aircraft (e.g. ELDORA)

For each of these modes a sweep is defined as follows:

- PPI mode: a sequence of rays at fixed elevation angle

- RHI mode: a sequence of rays at fixed azimuth angle

- pointing mode: a sequence of rays over some time period, at fixed azimuth and elevation

The volume may therefore be logically divided into sweeps. In CfRadial, we do not separate the sweeps in the stored field data arrays. Rather, we store arrays of start and stop indexes, which identify the rays that belong to each sweep. Some recorded rays may be in the transition region between defined sweeps, i.e. they may not belong to any sweep. For these rays we set the ‘antenna_transition’ flag to 1.

## Constant range geometry per volume

For a single volume, the CF/radial convention requires that the number of range bins, and the ranges for each of the bins, be non-varying in time. Therefore, within a volume

- the number of range bins is the same for all rays

- the range geometry is the same for all rays.

If the raw data range geometry changes over time within a volume, the data to be represented must be re-sampled using a common time-invariant range geometry for the volume.

## No grid mapping variable

The data in this format is saved in the native coordinate system for RADARs and LIDARs, i.e. radial (or polar) coordinates, with the instrument at the center.

A grid mapping type is not required, because the geo-reference variables contain all of the information required to locate the data in space.

For a stationary instrument, the following are stored as scalar variables (see section 4.5):

- latitude

- longitude

- altitude

Position and pointing references for moving platforms must take the following motions into account (see section 4.8):

- platform translation

- platform rotation

## Calibration information

Radars must be calibrated to ensure that the moments data are accurate. Calibration for some types of lidar may also be applicable.

A radar may have multiple sets of calibration parameters. Generally a separate calibration is performed for each transmit pulse width. Separate calibrations may be performed for other reasons as well. CfRadial supports storing multiple sets of calibration parameters, using the radar_calibration and lidar_calibration conventions.

The calibration applicable to a specific ray is indicated by the calibration_index variable.

## Compression

The netCDF 4 library supports files in the following formats:

- classic

- 64bit offset

- netcdf4

- netcdf4 classic

The netcdf4 format is built on HDF5, which supports compression. Where data are missing or unusable, the data values will be set to a constant well-known _FillValue code. This procedure, combined with the use of the netcdf4 format, provides efficient compression.

It is therefore recommended that the netcdf4 option be used whenever possible, to keep data sets as small as possible.

# Convention hierarchy

The CF/Radial convention comprises a base convention, along with a series of optional sub-conventions for specific purposes.

At the time of writing, the following conventions are envisaged:

| Convention name | Type | Description |
| --- | --- | --- |
| CF/Radial | Base | Radial data extension to the CF convention. Contains all necessary information for interpreting and displaying the data fields in a geo-referenced manner |
| instrument_parameters | Optional | Parameters common to both radar and lidar instruments |
| radar_parameters | Optional | Parameters specific to radars |
| lidar_parameters | Optional | Parameters specific to lidars |
| radar_calibration | Optional | Calibration values for radars |
| lidar_calibration | Optional | Calibration values for lidars |
| platform_velocity | Optional | Velocity of the platform, in multiple dimensions |
| geometry_correction | Optional | Corrections to the geometry of the data set |

If a netCDF file conforms to a base convention and one or more sub-conventions, these are concatenated in the Conventions attribute as a space-delimited string.

The following are some examples:

- “CF/Radial instrument_parameters”

- “CF/Radial instrument_parameters radar_parameters radar_calibration”

- “CF/Radial lidar_parameters platform_velocity”

# CF/Radial base convention

The base CF/Radial convention covers the minimum set of elements which are required to describe a radar/lidar data set sufficiently for basic display and plotting. CF/Radial is a specialization of CF.

NOTE on units: in the following tables, for conciseness, we do not spell out the units strings exactly as they are in the netCDF file. The following abbreviations apply:

| Units string in netCDF file | Abbreviation in tables |
| --- | --- |
| “degrees per second” | degrees/s |
| “meters per second” | m/s |

## Global attributes

| Attribute name | Type | Convention | Description |
| --- | --- | --- | --- |
| Conventions | string | CF | Conventions string will specify Cf/Radial, plus selected sub-conventions as applicable |
| title | string | CF | Short description of file contents |
| institution | string | CF | Where the original data were produced |
| references | string | CF | References that describe the data or the methods used to produce it |
| source | string | CF | Method of production of the original data |
| history | string | CF | List of modifications to the original data |
| comment | string | CF | Miscellaneous information |
| instrument_name | string | CF/Radial | Name of radar or lidar |
| site_name | string | CF/Radial | Name of site where data were gathered |
| scan_name | string | CF/Radial | Name of scan strategy used, if applicable |
| platform_is_mobile | string | CF/Radial | “true” or “false” |

## Dimensions

| Dimension name | Description |
| --- | --- |
| time | The number of rays. This dimension is optionally UNLIMITED |
| range | The number of range bins |
| sweep | The number of sweeps |
| frequency | Number of frequencies used |
| string_length ** | Length of char type variables. |

** Note: any number of ‘string_length’ dimensions may be created and used. For example, you may declare the dimensions ‘string_length”, ‘string_length_short’ and ‘string_length_long’, and use them appropriately for strings of various lengths. These are only used to indicate the length of the strings stored, and have no effect on other parts of the format.

## Global variables

| Variable name | Dimension | Type | Comments |
| --- | --- | --- | --- |
| volume_number | none | int | Volume numbers are sequential, relative to some arbitrary start time, and may wrap |
| platform_type | (string_length) | char | Options are: “fixed”, “vehicle”, “ship”, “aircraft”, “aircraft_fore”,“aircraft_aft”, “aircraft_tail”,“aircraft_belly”, “aircraft_roof”,“aircraft_nose”, “satellite_orbit”, “satellite_geostat” |
| instrument_type | (string_length) | char | Options are: “radar”, “lidar” |
| primary_axis | (string_length) | char | Options are: “axis_z”, “axis_y”, “axis_x” See section 7 for details. |
| time_coverage_start | (string_length) | char | UTC time of first ray in file. Format is: yyyy-mm-ddThh:mm:ssZ |
| time_coverage_end | (string_length) | char | UTC time of last ray in file. Format is: yyyy-mm-ddThh:mm:ssZ |

## Coordinate variables

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| time | (time) | double | seconds | Coordinate variable for time. Time at center of each ray, in fractional seconds since start_time. |
| range | (range) | float | meters | Coordinate variable for range. Range to center of each bin. |

### Attributes for time coordinate variable

| Attribute name | Type | Value |
| --- | --- | --- |
| standard_name | string | “time” |
| long_name | string | “time in seconds since volume start” |
| units | string | “seconds since yyyy-mm-ddThh:mm:ssZ”, where the actual start_time is stated |

### Attributes for range coordinate variable

| Attribute name | Type | Value |
| --- | --- | --- |
| standard_name | string | “projection_range_coordinate” |
| long_name | string | “range_to_measurement_volume” |
| units | string | “meters” |
| spacing_is_constant | string | “true” or “false” |
| meters_to_center_of_first_gate | float | Set to start range in meters |
| meters_between_gates | float | Set to gate spacing in meters Only applicable if spacing_is_constant is “true” |
| axis | string | “radial_range_coordinate” |

## Location variables

Note: for stationary platforms, these are scalars, and for moving platforms they are vectors with time.

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| latitude | none or (time) | double | degrees_north | Latitude of instrument. For a stationary platform, this is a scalar. For a moving platform, this is a vector. |
| longitude | none or (time) | double | degrees_east | Longitude of instrument. For a stationary platform, this is a scalar. For a moving platform, this is a vector. |
| altitude | none or (time) | double | meters | Altitude of instrument, above mean sea level. For a stationary platform, this is a scalar. For a moving platform, this is a vector. |
| altitude_agl | none or (time) | double | meters | Altitude of instrument above ground level. For a stationary platform, this is a scalar. For a moving platform, this is a vector. Omit if not known. |

## Sweep variables

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| sweep_number | (sweep) | int |  | The number of the sweep, in the volume scan. Starts at 0 each volume scan. |
| sweep_mode | (sweep, string_length) | char |  | Options are: “sector”,“coplane”,rhi”, “vertical_pointing”,“idle”, “azimuth_surveillance”, “elevation_surveillance”, “sunscan”,“pointing”, “manual_ppi”,“manual_rhi” |
| fixed_angle | (sweep) | float | degrees | Target angle for the sweep. elevation in most modes azimuth in RHI mode |
| sweep_start_ray_index | (sweep) | int |  | Index of first ray in sweep, relative to start of volume. 0-based. |
| sweep_end_ray_index | (sweep) | int |  | Index of last ray in sweep, relative to start of volume. 0-based. |
| target_scan_rate | (sweep) | float | degrees/s | Intended scan rate for this sweep. The actual scan rate is stored according to section 4.7. This variable is optional. Omit if not available. |

NOTE: this section must always exist, even if a volume contains only a single sweep. The reason is that the sweep_mode and sweep_fixed_angle are necessary for fully understanding the sweep strategy.

## Sensor pointing variables

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| azimuth | (time) | float | degrees | Azimuth of antenna, relative to true north. |
| elevation | (time) | float | degrees | Elevation of antenna, relative to the horizontal plane. |
| scan_rate | (time) | float | degrees/s | Antenna scan rate. Set to negative if counter-clockwise in azimuth or decreasing in elevation. Positive otherwise. Omit if not known. |
| antenna_transition | (time) | byte |  | 1 if antenna is in transition, i.e. between sweeps, 0 if not. Omit if not known. If variable is omitted, the transition will be assumed to be 0 everywhere. |

### Attributes for azimuth(time) variable

| Attribute name | Type | Value |
| --- | --- | --- |
| standard_name | string | “azimuth angle” |
| long_name | string | “azimuth angle from true north” |
| units | string | “degrees” |
| axis | string | “radial_azimuth_coordinate” |

### Attributes for elevation(time) variable

| Attribute name | Type | Value |
| --- | --- | --- |
| standard_name | string | “elevation angle” |
| long_name | string | “elevation angle from horizontal” |
| units | string | “degrees” |
| axis | string | “radial_elevation_coordinate” |

## Moving platform geo-reference variables

For moving platforms, the following additional variables will be included to allow geo-referencing of the platform in earth coordinates. See section 7 for further details.

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| heading | (time) | float | degrees | Heading of the platform relative to true N, looking down from above. |
| roll | (time) | float | degrees | Roll about longitudinal axis of platform. Positive is left side up, looking forward. |
| pitch | (time) | float | degrees | Pitch about the lateral axis of the platform. Positive is up at the front. |
| drift | (time) | float | degrees | Difference between heading and track over the ground. Positive drift implies track is clockwise from heading, looking from above. NOTE: not applicable to land-based moving platforms. |
| rotation | (time) | float | degrees | Angle between the radar beam and the vertical axis of the platform. Zero is along the vertical axis, positive is clockwise looking forward from behind the platform. |
| tilt | (time) | float | degrees | Angle between radar beam (when it is in a plane containing the longitudinal axis of the platform) and a line perpendicular to the longitudinal axis. Zero is perpendicular to the longitudinal axis, positive is towards the front of the platform. |

## Moments field data variables

Each 2-D moments field variable has the dimensions of (time, range).

The field data will be stored using one of the following:

| netCDF type | Byte width | Description |
| --- | --- | --- |
| ncbyte | 1 | scaled signed integer |
| short | 2 | scaled signed integer |
| int | 4 | scaled signed integer |
| float | 4 | floating point |
| double | 8 | floating point |

The netCDF variable name is interpreted as the short name for the field.

Field data variables have the following attributes:

| Attribute name | Type | Convention | Description |
| --- | --- | --- | --- |
| long_name | string | CF | Longer name describing the field |
| standard_name | string | CF | CF standard name for field |
| units | string | CF | Units for field |
| _FillValue | same type as field data | CF | Used if data are missing at this range bin |
| scale_factor | float | CF | Float value = (integer value) * scale_factor + add_offset |
| add_offset | float | CF |  |
| coordinates | string | CF | See note below |

NOTE: the “coordinates’ attribute lists the variables needed to compute the location of a data point in space.

For stationary platforms, the coordinates attribute should be set to:

“elevation azimuth range”

For moving platforms, the coordinates attribute should be set to:

“elevation azimuth range heading roll pitch rotation tilt”

# Sub-conventions

The base CF/Radial convention, as described above, covers the minimum set of netCDF elements which are required to locate radar/lidar data in time and space.

The following sub-conventions augment the base convention with additional information for various purposes.

## The instrument_parameters sub-convention

This convention stores parameters relevant to both radars and lidars.

Variables in this convention will have the string attribute meta_group, set to the value “instrument_parameters”.

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| frequency | (frequency) | float | Hz | List of operating frequencies, in Hertz. In most cases, only a single frequency is used. |
| follow_mode | (sweep, string_length) | char |  | options are: “none”, “sun”, “vehicle”, “aircraft”, “target”, “manual” |
| pulse_width | (time) | float | seconds |  |
| prt_mode | (sweep, string_length) | char |  | Pulsing mode Options are: “fixed”, “staggered”, “dual” |
| prt | (sweep) if prt constant (time) if dual prt (sweep, prt) if staggered prt | float | seconds | Pulse repetition time. For fixed prt, this is set per sweep. For dual prt, this is set per time, because prt changes every ray. For staggered prt, for each sweep list the prt values used, using the prt dimension. |
| prt_ratio | (sweep) | float |  | For dual/staggered prt mode. This is set per sweep. |
| polarization_mode | (sweep, string_length) | char |  | Options are: “horizontal”, “vertical”, “hv_alt”, “hv_sim”, “circular” |
| nyquist_velocity | (time) | float | m/s | Unambiguous velocity |
| unambiguous_range | (time) | float | meters | Unambiguous range |
| scan_rate | (time) | float | degrees/s | Antenna scan rate Set to missing if not available. |
| n_samples | (time) | int |  | Number of samples used to compute moments |

The instrument_parameters convention also specifies one extra, but optional, attribute – sampling_ratio - for each field variable.

The number of samples used to compute the moments may vary from field to field. In the table above, n_samples refers to the maximum number of samples used for any field. The sampling_ratio is computed as the actual number of samples used for any field, divided by n_samples.

| Attribute name | Type | Description |
| --- | --- | --- |
| sampling_ratio | float | n_samples for this field divided by n_samples specified for each time (see table above) |

If this attribute is missing, its value will be assumed to be 1.0.

## The radar_parameters sub-convention

This convention handles parameters specific to radar platforms. Variables in this convention will have the string attribute meta_group, set to the value “radar_parameters”.

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| radar_antenna_gain_h | none | float | dB | Nominal antenna gain, H polarization |
| radar_antenna_gain_v | none | float | dB | Nominal antenna gain, V polarization |
| radar_beam_width_h | none | float | degrees | Antenna beam width H polarization |
| radar_beam_width_v | none | float | degrees | Antenna beam width V polarization |
| radar_measured_transmit_power_h | (time) | float | dBm | Measured transmit power H polarization |
| radar_measured_transmit_power_v | (time) | float | dBm | Measured transmit power V polarization |

## The lidar_parameters sub-convention

This convention handles parameters specific to lidar platforms. Variables in this convention will have the string attribute meta_group, set to the value “lidar_parameters”.

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| lidar_beam_divergence | none | float | milliradians | Transmit side |
| lidar_field_of_view | none | float | milliradians | Receive side |
| lidar_aperture_diameter | none | float | cm |  |
| lidar_aperture_efficiency | none | float | percent |  |
| lidar_peak_power | none | float | watts |  |
| lidar_pulse_energy | none | float | joules |  |

## The radar_calibration sub-convention

Variables in this convention will have the string attribute meta_group, set to the convention name “radar_calibration”.

### Dimensions

| Dimension name | Description |
| --- | --- |
| r_calib | The number of calibrations available |

### Variables

The meaning of the designations used in the calibration variables are as follows for dual-polarization radars:

- 'h': horizontal channel

- 'v': vertical channel

- 'hc': horizontal co-polar (h transmit, h receive)

- 'hx' – horizontal cross-polar (v transmit, h receive)

- 'vc': vertical co-polar (v transmit, v receive)

- 'vx' – vertical cross-polar (h transmit, v receive)

For single polarization radars, the 'h' quantities should be used.

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| r_calib_index | (time) | byte |  | index for the calibration that applies to each ray |
| r_calib_time | (r_calib, string_length) | char | UTC | e.g. 2008-09-25 T23:00:00Z |
| r_calib_pulse_width | (r_calib) | float | seconds | Pulse width for this calibration |
| r_calib_receiver_bandwidth | (r_calib) | float | MHz | Bandwidth of receiver, nominally: 1.0 / pulse_width |
| r_calib_ant_gain_h | (r_calib) | float | dB | Derived antenna gain H channel |
| r_calib_ant_gain_v | (r_calib) | float | dB | ditto, V channel |
| r_calib_xmit_power_h | (r_calib) | float | dBm | Transmit power H channel |
| r_calib_xmit_power_v | (r_calib) | float | dBm | ditto, V channel |
| r_calib_two_way_waveguide_loss_h | (r_calib) | float | dB | 2-way waveguide loss measurement plane to feed horn H channel |
| r_calib_two_way_waveguide_loss_v | (r_calib) | float | dB | ditto, V channel |
| r_calib_two_way_radome_loss_h | (r_calib) | float | dB | 2-way radome loss H channel |
| r_calib_two_way_radome_loss_v | (r_calib) | float | dB | ditto, V channel |
| r_calib_receiver_mismatch_loss | (r_calib) | float | dB | Receiver filter bandwidth mismatch loss |
| r_calib_radar_constant_h | (r_calib) | float | m/mW dB units | Radar constant H channel |
| r_calib_radar_constant_v | (r_calib) | float | m/mW dB units | ditto, V channel |
| r_calib_noise_hc | (r_calib) | float | dBm | Measured noise level H co-pol channel |
| r_calib_noise_vc | (r_calib) | float | dBm | ditto, V co-pol channel |
| r_calib_noise_hx | (r_calib) | float | dBm | ditto, H cross-pol |
| r_calib_noise_vx | (r_calib) | float | dBm | ditto, V cross-pol |
| r_calib_receiver_gain_hc | (r_calib) | float | dB | Measured receiver gain H co-pol channel |
| r_calib_receiver_gain_vc | (r_calib) | float | dB | ditto, V co-pol channel |
| r_calib_receiver_gain_hx | (r_calib) | float | dB | ditto, H cross-pol |
| r_calib_receiver_gain_vx | (r_calib) | float | dB | ditto, V cross-pol |
| r_calib_base_1km_hc | (r_calib) | float | dBZ | reflectivity at 1km for SNR=0dB H co-pol channel |
| r_calib_base_1km_vc | (r_calib) | float | dBZ | ditto, V co-pol channel |
| r_calib_base_1km_hx | (r_calib) | float | dBZ | ditto, H cross-pol |
| r_calib_base_1km_vx | (r_calib) | float | dBZ | ditto, V cross-pol |
| r_calib_sun_power_hc | (r_calib) | float | dBm | Calibrate sun power H co-pol channel |
| r_calib_sun_power_vc | (r_calib) | float | dBm | ditto, V co-pol channel |
| r_calib_sun_power_hx | (r_calib) | float | dBm | ditto, H cross-pol |
| r_calib_sun_power_vx | (r_calib) | float | dBm | ditto, V cross-pol |
| r_calib_noise_source_power_h | (r_calib) | float | dBm | Noise source power H channel |
| r_calib_noise_source_power_v | (r_calib) | float | dBm | ditto, V channel |
| r_calib_power_measure_loss_h | (r_calib) | float | dB | Power measurement loss in coax and connectors H channel |
| r_calib_power_measure_loss_v | (r_calib) | float | dB | ditto, V channel |
| r_calib_coupler_forward_loss_h | (r_calib) | float | dB | Coupler loss into waveguide H channel |
| r_calib_coupler_forward_loss_v | (r_calib) | float | dB | ditto, V channel |
| r_calib_zdr_correction | (r_calib) | float | dB | corrected = measured + correction |
| r_calib_ldr_correction_h | (r_calib) | float | dB | corrected = measured + correction |
| r_calib_ldr_correction_v | (r_calib) | float | dB | corrected = measured + correction |
| r_calib_system_phidp | (r_calib) | float | degrees | System PhiDp, as seen in drizzle close to radar |
| r_calib_test_power_h | (r_calib) | float | dBm | Calibration test power H channel |
| r_calib_test_power_v | (r_calib) | float | dBm | ditto, V channel |
| r_calib_receiver_slope_hc | (r_calib) | float |  | Computed receiver slope, ideally 1.0 H co-pol channel |
| r_calib_receiver_slope_vc | (r_calib) | float |  | ditto, V co-pol channel |
| r_calib_receiver_slope_hx | (r_calib) | float |  | ditto, H cross-pol |
| r_calib_receiver_slope_vx | (r_calib) | float |  | ditto, V cross-pol |

## The lidar_calibration sub-convention

Variables in this convention will have the string attribute meta_group, set to the value “lidar_calibration”.

At the time of writing, this convention has not been defined.

## The platform_velocity sub-convention

For moving platforms, the following additional variables will be included to indicate the velocity of the platform at each time.

Variables in this convention will have the string attribute meta_group, set to the value “platform_velocity”.

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| eastward_velocity | (time) | float | m/s | EW velocity of the platform. Positive is eastwards. |
| northward_velocity | (time) | float | m/s | NS velocity of the platform. Positive is northwards. |
| vertical_velocity | (time) | float | m/s | Vertical velocity of the platform. Positive is up. |
| eastward_wind | (time) | float | m/s | EW wind at the platform location. Positive is eastwards. |
| northward_wind | (time) | float | m/s | NS wind at the platform location. Positive is northwards. |
| vertical_wind | (time) | float | m/s | Vertical wind at the platform location. Positive is up. |
| heading_rate | (time) | float | degrees/s | Rate of change of heading |
| roll_rate | (time) | float | degrees/2 | Rate of change of roll of the platform |
| pitch_rate | (time) | float | degrees/s | Rate of change of pitch of the platform. |

## The geometry_correction sub-convention

The following additional variables will be included to quantify errors in the georeference data for the platform. These are constant for a data set.

Variables in this convention will have the string attribute meta_group, set to the value “geometry_correction”.

| Variable name | Dimension | Type | Units | Comments |
| --- | --- | --- | --- | --- |
| azimuth_correction | none | float | degrees | correction to azimuth values |
| elevation_correction | none | float | degrees | correction to elevation values |
| range_correction | none | float | degrees | correction to range values |
| longitude_correction | none | float | degrees | correction to longitude values |
| latitude_correction | none | float | degrees | correction to latitude values |
| pressure_altitude_correction | none | float | meters | correction to pressure altitude values |
| radar_altitude_correction | none | float | meters | correction to radar altitude values |
| eastward_ground_speed_correction | none | float | m/s | correction to EW ground speed values |
| northward_ground_speed_correction | none | float | m/s | correction to NS ground speed values |
| vertical_velocity_correction | none | float | m/s | correction to vertical velocity values |
| heading_correction | none | float | degrees | correction to heading values |
| roll_correction | none | float | degrees | correction to roll values |
| pitch_correction | none | float | degrees | correction to pitch values |
| drift_correction | none | float | degrees | correction to drift values |
| rotation_correction | none | float | degrees | correction to rotation values |
| tilt_correction | none | float | degrees | correction to tilt values |

# Standard names

To the extent possible, CfRadial uses standard names already defined by CF.

Section 6.1 lists the proposed standard names for metadata variables, and section 6.2 lists the proposed standard names for moments data.

## Proposed standard names for metadata variables

| Variable name Standard name | Units | Already supported in CF? |
| --- | --- | --- |
| altitude_agl altitude_above_ground_level | meters | no |
| altitude_correction altitude_correction | meters | no |
| altitude altitude | meters | yes |
| antenna_transition antenna_is_in_transition_between_sweeps | unitless | no |
| azimuth_correction azimuth_angle_correction | degrees | no |
| azimuth beam_azimuth_angle | degrees | no |
| drift_correction platform_drift_angle_correction | degrees | no |
| drift platform_drift_angle | degrees | no |
| eastward_velocity_correction platform_eastward_velocity_correction | m/s | no |
| eastward_velocity platform_eastward_velocity | m/s | no |
| eastward_wind eastward_wind_speed | m/s | yes |
| elevation_correction beam_elevation_angle_correction | degrees | no |
| elevation beam_elevation_angle | degrees | no |
| time_coverage_end data_volume_end_time_utc | seconds | no |
| fixed_angle target_fixed_angle | degrees | no |
| follow_mode follow_mode_for_scan_strategy | unitless | no |
| frequency radiation_frequency | s-1 | no |
| heading_change_rate platform_heading_angle_rate_of_change | degrees | no |
| heading_correction platform_heading_angle_correction | degrees | no |
| heading platform_heading_angle | degrees | no |
| instrument_name name_of_instrument | unitless | no |
| instrument_type type_of_instrument | unitless | no |
| latitude_correction latitude_correction | degrees | no |
| latitude latitude | degrees_east | no |
| lidar_aperture_diameter lidar_aperture_diameter | meters | no |
| lidar_aperture_efficiency lidar_aperture_efficiency | unitless | no |
| lidar_beam_divergence lidar_beam_divergence | radians | no |
| lidar_constant lidar_calibration_constant | unitless | no |
| lidar_field_of_view lidar_field_of_view | radians | no |
| lidar_peak_power lidar_peak_power | watts | no |
| lidar_pulse_energy lidar_pulse_energy | joules | no |
| longitude_correction longitude_correction | degrees | no |
| longitude longitude | degrees_east | no |
| northward_velocity_correction platform_northward_velocity_correction | m/s | no |
| northward_velocity platform_northward_velocity | m/s | no |
| northward_wind northward_wind | m/s | yes |
| nyquist_velocity unambiguous_doppler_velocity | m/s | no |
| n_samples number_of_samples_used_to_compute_moments | unitless | no |
| pitch_change_rate platform_pitch_angle_rate_of_change | degrees | no |
| pitch_correction platform_pitch_angle_correction | degrees | no |
| pitch platform_pitch_angle | degrees | yes |
| platform_is_mobile platform_is_mobile | unitless | no |
| platform_type platform_type | unitless | no |
| polarization_mode transmit_receive_polarization_mode | unitless | no |
| prt_mode transmit_pulse_mode | unitless | no |
| pressure_altitude_correction pressure_altitude_correction | meters | no |
| primary_axis primary_axis_of_rotation | unitless | no |
| prt pulse_repetition_frequency | /s | no |
| prt_ratio multiple_pulse_repetition_frequency_ratio |  | no |
| pulse_width transmitter_pulse_width | seconds | no |
| radar_antenna_gain_h nominal_radar_antenna_gain_h_channel | dB | no |
| radar_antenna_gain_v nominal_radar_antenna_gain_v_channel | dB | no |
| radar_beam_width_h half_power_radar_beam_width_h_channel | degrees | no |
| radar_beam_width_v half_power_radar_beam_width_v_channel | degrees | no |
| radar_transmit_power_h radar_transmit_power_h_channel | dBm | no |
| radar_transmit_power_v radar_transmit_power_v_channel | dBm | no |
| range_correction range_to_center_of_measurement_volume_correction | meters | no |
| range range_to_center_of_measurement_volume | meters | no |
| roll_correction platform_roll_angle_correction | degrees | no |
| roll platform_roll_angle | degrees | yes |
| rotation_correction beam_rotation_angle_relative_to_platform_correction | degrees | no |
| rotation beam_rotation_angle_relative_to_platform | degrees | no |
| r_calib_antenna_gain_h calibrated_radar_antenna_gain_h_channel | dB | no |
| r_calib_antenna_gain_v calibrated_radar_antenna_gain_v_channel | dB | no |
| r_calib_base_dbz_1km_hc radar_reflectivity_at_1km_at_zero_snr_h_co_polar_channel | dBZ | no |
| r_calib_base_dbz_1km_hx radar_reflectivity_at_1km_at_zero_snr_h_cross_polar_channel | dBZ | no |
| r_calib_base_dbz_1km_vc radar_reflectivity_at_1km_at_zero_snr_v_co_polar_channel | dBZ | no |
| r_calib_base_dbz_1km_vx radar_reflectivity_at_1km_at_zero_snr_v_cross_polar_channel | dBZ | no |
| r_calib_coupler_forward_loss_h radar_calibration_coupler_forward_loss_h_channel | dB | no |
| r_calib_coupler_forward_loss_v radar_calibration_coupler_forward_loss_v_channel | dB | no |
| r_calib_index calibration_data_array_index_per_ray | unitless | no |
| r_calib_ldr_correction_h calibrated_radar_ldr_correction_h_channel | dB | no |
| r_calib_ldr_correction_v calibrated_radar_ldr_correction_v_channel | dB | no |
| r_calib_noise_hc calibrated_radar_receiver_noise_h_co_polar_channel | dBm | no |
| r_calib_noise_hx calibrated_radar_receiver_noise_h_cross_polar_channel | dBm | no |
| r_calib_noise_vc calibrated_radar_receiver_noise_v_co_polar_channel | dBm | no |
| r_calib_noise_vx calibrated_radar_receiver_noise_v_cross_polar_channel | dBm | no |
| r_calib_noise_source_power_h radar_calibration_noise_source_power_h_channel | dBm | no |
| r_calib_noise_source_power_v radar_calibration_noise_source_power_v_channel | dBm | no |
| r_calib_power_measure_loss_h radar_calibration_power_measurement_loss_h_channel | dB | no |
| r_calib_power_measure_loss_v radar_calibration_power_measurement_loss_v_channel | dB | no |
| r_calib_pulse_width radar_calibration_pulse_width | seconds | no |
| r_calib_radar_constant_h calibrated_radar_constant_h_channel | (m/mW)dB | no |
| r_calib_radar_constant_v calibrated_radar_constant_v_channel | (m/mW)dB | no |
| r_calib_receiver_gain_hc calibrated_radar_receiver_gain_h_co_polar_channel | dB | no |
| r_calib_receiver_gain_hx calibrated_radar_receiver_gain_h_cross_polar_channel | dB | no |
| r_calib_receiver_gain_vc calibrated_radar_receiver_gain_v_co_polar_channel | dB | no |
| r_calib_receiver_gain_vx calibrated_radar_receiver_gain_v_cross_polar_channel | dB | no |
| r_calib_receiver_mismatch_loss radar_calibration_receiver_mismatch_loss | dB | no |
| r_calib_receiver_slope_hc calibrated_radar_receiver_slope_h_co_polar_channel | unitless | no |
| r_calib_receiver_slope_hx calibrated_radar_receiver_slope_h_cross_polar_channel | unitless | no |
| r_calib_receiver_slope_vc calibrated_radar_receiver_slope_v_co_polar_channel | unitless | no |
| r_calib_receiver_slope_vx calibrated_radar_receiver_slope_v_cross_polar_channel | unitless | no |
| r_calib_sun_power_hc calibrated_radar_sun_power_h_co_polar_channel | dBm | no |
| r_calib_sun_power_hx calibrated_radar_sun_power_h_cross_polar_channel | dBm | no |
| r_calib_sun_power_vc calibrated_radar_sun_power_v_co_polar_channel | dBm | no |
| r_calib_sun_power_vx calibrated_radar_sun_power_v_cross_polar_channel | dBm | no |
| r_calib_system_phidp calibrated_radar_system_phidp | degrees | no |
| r_calib_test_power_h radar_calibration_test_power_h_channel | dBm | no |
| r_calib_test_power_v radar_calibration_test_power_v_channel | dBm | no |
| r_calib_time radar_calibration_time_utc | unitless | no |
| r_calib_two_way_radome_loss_h radar_calibration_two_way_radome_loss_h_channel | dB | no |
| r_calib_two_way_radome_loss_v radar_calibration_two_way_radome_loss_v_channel | dB | no |
| r_calib_two_way_waveguide_loss_h radar_calibration_two_way_waveguide_loss_h_channel | dB | no |
| r_calib_two_way_waveguide_loss_v radar_calibration_two_way_waveguide_loss_v_channel | dB | no |
| r_calib_xmit_power_h calibrated_radar_xmit_power_h_channel | dBm | no |
| r_calib_xmit_power_v calibrated_radar_xmit_power_v_channel | dBm | no |
| r_calib_zdr_correction calibrated_radar_zdr_correction | dB | no |
| scan_name name_of_antenna_scan_strategy | unitless | no |
| scan_rate antenna_angle_scan_rate | unitless | no |
| site_name name_of_instrument_site | unitless | no |
| spacing_is_constant spacing_between_range_gates_is_constant | unitless | no |
| sweep_end_ray_index index_of_last_ray_in_sweep | unitless | no |
| sweep_mode scan_mode_for_sweep | unitless | no |
| sweep_number sweep_index_number_0_based | unitless | no |
| sweep_start_ray_index index_of_first_ray_in_sweep | unitless | no |
| sweep_unambiguous_range unambiguous_range_for_sweep | meters | no |
| threshold_field_name name_of_data_field_for_thresholding | unitless | no |
| threshold_value value_applied_to_threshold_field | unitless | no |
| tilt_correction beam_tilt_angle_relative_to_platform_correction | degrees | no |
| tilt beam_tilt_angle_relative_to_platform | degrees | no |
| time time | seconds | no |
| time_coverage_start data_volume_start_time_utc | unitless | no |
| unambiguous_range unambiguous_range | meters | no |
| vertical_velocity_correction platform_vertical_velocity_correction | m/s | no |
| vertical_velocity platform_vertical_velocity | m/s | no |
| vertical_wind upward_air_velocity | m/s | yes |
| volume_number data_volume_index_number | unitless | no |

## Standard names for moments variables

| Standard name | Short name | Units | Already in CF? |
| --- | --- | --- | --- |
| equivalent_reflectivity_factor |  | dBZ | yes |
| linear_equivalent_reflectivity_factor |  | Z | no |
| radial_velocity_of_scatterers_away_from_instrument |  | m/s | yes |
| spectrum_width |  | m/s | no |
| log_differential_reflectivity_hv | ZDR | dB | no |
| log_linear_depolarization_ratio_hv | LDR | dB | no |
| log_linear_depolarization_ratio_h | LDRH | dB | no |
| log_linear_depolarization_ratio_v | LDRV | dB | no |
| differential_phase_hv | PHIDP | degrees | no |
| specific_differential_phase_hv | KDP | degrees/km | no |
| cross_correlation_ratio_hv | RHOHV |  | no |
| log_power | DBM | dBm | no |
| log_power_co_polar_h | DBMHC | dBm | no |
| log_power_cross_polar_h | DBMHX | dBm | no |
| log_power_co_polar_v | DBMVC | dBm | no |
| log_power_cross_polar_v | DBMVX | dBm | no |
| linear_power | PWR | mW | no |
| linear_power_co_polar_h | PWRHC | mW | no |
| linear_power_cross_polar_h | PWRHX | mW | no |
| linear_power_co_polar_v | PWRVC | mW | no |
| linear_power_cross_polar_v | PWRVX | mW | no |
| signal_to_noise_ratio | SNR | dB | no |
| signal_to_noise_ratio_co_polar_h | SNRHC | dB | no |
| signal_to_noise_ratio_cross_polar_h | SNRHX | dB | no |
| signal_to_noise_ratio_co_polar_v | SNRVC | dB | no |
| signal_to_noise_ratio_cross_copolar_v | SNRVX | dB | no |
| normalized_coherent_power | NCP |  | no |

# Computing gate location from geo-reference variables

## Symbol definitions

Χa: platform relative coordinate system (xa, ya, za) – see figure 7.1

Χh: coordinate system relative to level platform (no roll or pitch) with heading H.

Χ: earth-relative coordinate system (x, y, z), x is positive east, y is positive north, z is positive up.

H: heading of platform (see figure 7.2)

T: track of platform (see figure 7.2)

D: drift angle (see figure 7.2)

P: pitch angle (see figure 7.2)

R: roll angle (see figure 7.2)

λ: azimuth angle

: elevation angle

θ: rotation angle

τ: tilt angle

r: range

h: height

h0: height of the instrument

R’: pseudo radius of earth =

## Stationary platforms

For stationary platforms, it is assumed that the elevation angle is measured relative to the horizontal plane and the azimuth angle is measured clockwise from true north.

### LIDARs

For LIDARs, the assumption is generally made that propagation of the beam is in a straight line, emanating at the instrument. Location computations are performed assuming a Cartesian coordinate system:

where

x is positive east

y is positive north

(x0, y0, z0)  are the coordinates of the instrument.

Remember that azimuth is the angle clockwise from true north.

The instrument location is specified in longitude, latitude and altitude. Locations in the earth’s spherical coordinate system are computed using the instrument location and the (x,y,z) from above, using normal spherical geometry.

### RADARs

The propagation of radar microwave energy in a beam through the low atmosphere is affected by the change of refractive index of the atmosphere with height. Under average conditions this causes the beam to be deflected downwards, in what is termed ‘Standard Refraction’. For most purposes this is adequately modeled by assuming that the beam is in fact straight, relative to an earth which has a radius of 4/3 times the actual earth radius. (Rinehart 2004.)

For a stationary, ground-based radar, the equations are similar to those for the LIDAR case, except that we have one extra term, the height correction, which reflects the beam curvature relative to the earth.

The height above the earth’s surface for a given range is:

See Rinehart 2004, chapter 3, for more details.

The (x,y) location for a given range is:

where x is positive east, y is positive north, and remembering that azimuth is the angle clockwise from true north.

## Moving platforms

For moving platforms, the metadata for each beam will include:

- longitude of instrument

- latitude of instrument

- altitude of instrument

- rotation and tilt of the beam (see section 7)

- roll, pitch and heading of the platform

We use the same equations as for stationary platforms. However, since elevation and azimuth angle are not directly measured, these must be computed from rotation, tilt, roll, pitch and heading.

Note that for airborne radar platforms, correcting for refractive index does not apply. Therefore, for airborne radars, use the straight line equations for LIDARs.

Refer to section 7.4 for the computation of elevation and azimuth relative to earth coordinates.

Then apply the following equations, as before, to compute the location of the observed point.

## The geometry of moving platforms

### Reference frame

NOTE: -see Lee et al. (1994) for further background on this topic, and on the corrections to Doppler velocity for moving platforms.

Figure 7.1 depicts the theoretical reference frame for a moving platform, such as an aircraft. This discussion also applies to water-borne platforms and land-based moving platforms, but we will use the aircraft analogy here.

Figure 7.1: Moving platform axis definitions and reference frame
(reproduced from Lee et al., 1994,
originally from Axford, 1968)
(c)American Meteorological Society. Reprinted with permission.

We use right-handed coordinate systems in this discussion. Angles are positive for clockwise rotations about an axis perpendicular to the plane when looking away from the origin.

We consider three coordinate systems here:

- Χa, coordinates relative to the platform

- Χh, coordinates relative to a platform with 0 roll and 0 pitch, but with the Y axis aligned with the aircraft heading

- Χ, relative to the earth, +y points north and +x points east.

- The radar pointing angles rotation (θ) and tilt (τ) are measured relative to Χa. The requirement is to derive the angles elevation (φ) and azimuth (λ) relative to Χ.

The airframe-relative coordinate system is characterized by 3 angles: pitch (P), roll (R) and heading (H). These angles are generally measured by an inertial navigation system (INS).

The platform moves relative to X, based on its heading H, and the drift D, caused by wind or current. (D is not relevant to land-based platforms). The track T is the line of movement over the ground.

Figures 6.2 a through c show the definitions of heading, drift, pitch and roll.

Figure 7.2(a): Definition of heading, drift and track.
Reproduced from Lee et al., 1994.
(c)American Meteorological Society. Reprinted with permission.

Figure 7.2(b): Definition of pitch

Figure 7.2(c): Definition of roll

### Primary rotation axes

Weather radars and lidars rotate primarily about a principal axis. The antenna can also slew about a secondary axis, orthogonal to the primary axis.

In the case of most fixed radars, this principle axis is generally vertical. The azimuth angle (λ) is measured clockwise from true north, and the elevation angle (φ) is measured as positive above the horizontal plane and negative below it.

With moving platforms, we maintain that same definition for azimuth and elevation , i.e. that they are relative to X, the earth reference frame. Our task here is to derive azimuth and elevation from angular measurements made relative to the platform, and from the platform reference frame itself.

We define two additional terms, rotation (θ) and tilt(τ), which are angular measurements made relative to the platform.

The conversion from rotation and tilt to elevation and azimuth depends on the primary axis of rotation.

We use the global variable “primary_axis” to indicate in a CfRadial file which axis is primary.

#### Type Z radars: primary axis za (normal radar, nose radar)

If the primary axis is za, the rotation angle θ is 0 in the (ya, za) plane and the tilt angle (τ) is 0 in the (xa, ya) plane.

The location of a gate in Χa coordinates is:

#### Type Y radars: primary axis ya (e.g. tail radar rotating around longitudinal axis of fuselage, ELDORA)

If the primary axis is ya, the rotation angle θ is 0 in the (ya, za) plane and the tilt angle (τ) is 0 in the (xa, za) plane.

The location of a gate in Χa coordinates is:

#### Type X radars, primary axis xa (e.g. belly radar scanning forward and aft, EDOP)

If the primary axis is xa, the rotation angle θ is 0 in the (xa, za) plane and the tilt angle (τ) is 0 in the (ya, za) plane.

The location of a gate in Χa coordinates is:

### Rotating Xa to Xh

Rotating Xa to Xh requires 2 steps:

- first remove the roll R, by rotating the x axis around the y axis by –R.

- then remove the pitch P, by rotating the y axis around the x axis by –P.

The transformation matrix for removing the roll component is:

The transformation matrix for removing the pitch component is:

We apply these transformations consecutively:

For type Z radars:

For type Y radars:

For type Z radars:

### Computing earth-relative elevation and azimuth

After we have applied the above transformations to obtain coordinates relative to Xh, we can compute the azimuth and elevation as follows:

# Change log

## Version 1.0, draft7: 2010-06-16

This was the first version posted to the web.

## Version 1.0, draft8: 2010-11-01

The following changes were made for draft 8:

Changes to section 1.4

Changed missing_value to _FillValue.

Changes to section 2.6

Removed the use of the grid mapping variable, since this is not really necessary for this format. The location information is provided in the location variables and is independent of any grid mapping.

Changes to section 3

Removed the ‘frequency_list’ sub-convention. The list of frequencies used is now a required member of the ‘instrument_parameters’ sub-convention.

Changes to section 4.2

Relaxed the requirement to have a single dimension to handle string length. Any number of dimensions may be added for strings of different lengths. Readers of the data must check the dimension used by each string variable to determine its length.

The ‘frequency’ dimension is now required if the ‘instrument_parameters’ sub-convention is used. The wavelength variable has been removed, and is replaced by a list of 1 or more frequencies.

Changes to section 4.3

Added “aircraft” to the platform type list.

Moved ‘instrument_type’ up into the base convention, from the ‘instrument_params’ sub-convention.

Changed start_time variable to time_coverage_start, and end_time to time_coverage_end, to conform to the NetCDF Attribute Convention for Dataset Discovery (ACDD).

Changes to section 4.5

‘altitude_agl’ is now an optional variable. Omit this variable if not known.

Changes to section 4.6

Changed ‘sweep_fixed_angle’ to ‘fixed_angle’.

Added ‘target_scan_rate’ as an optional variable. Omit this variable if not known.

Changes to section 4.7

Changed ‘antenna_transition’to be optional. Omit this variable if not known. It will then be assumed that the antenna is not in transition for any ray.

Added ‘scan_rate’ variable, which is optional. Omit this variable if the values are not known.

Changes to section 4.9

Changed missing_value to _FillValue.

Changes to section 5.1

Changed sweep_follow_mode to follow_mode.

Changed sweep_prf_mode to prt_mode.

Added prt_ratio.

Changed sweep_polarization_mode to polarization_mode.

Changes to section 5.2

Changed radar_transmit_power_h to radar_measured_transmit_power_h.

Changed radar_transmit_power_v to radar_measured_transmit_power_v.

Moved radar_receiver_bandwidth into 5.4.2.

Removed Changewavelength. Only frequencies are specified.

Added the option for more than 2 PRTs.

Relaxed the requirements – if a variable is not specified, a default value will be assumed. These are specified in the document.

Added linear reflectivity (Z) as a field with a proposed standard name.

Calibration as a variable with attributes?

Radar rotation direction? Sign to indicate dirn?

# References

Axford, D. N., 1968: On the accuracy of wind measurements using an inertial platform in an aircraft, and an example of a measurement of the vertical structure of the atmosphere. J. Appl. Meteor., 7, 645-666.

Lee, W., P. Dodge, F. D. Marks Jr. and P. Hildebrand, 1994: Mapping of Airborne Doppler Radar Data. Journal of Oceanic and Atmospheric Technology, 11, 572 – 578.

Rinehart, R. E., 2004: Radar for Meteorologists, Fourth Edition. Rinehart Publications. ISBN 0-9658002-1-0